指标概述：

Figure 1. Block diagram of WR34manifold

Table 1 Frequency list of WR34 Manifold

| Ch.Index | port No. | Centre Fre. | BW |
| --- | --- | --- | --- |
| ch1 | J8 | 27725 MHz | 236 MHz |
| ch2 | J1 | 27975 MHz | 236 MHz |
| ch3 | J7 | 28225 MHz | 236 MHz |
| ch4 | J2 | 28475 MHz | 236 MHz |
| ch5 | J6 | 28725 MHz | 236 MHz |
| ch6 | J3 | 28975 MHz | 236 MHz |
| ch7 | J5 | 29625 MHz | 236 MHz |
| ch8 | J4 | 29875 MHz | 236 MHz |

Table 2 Specifications of WR34 Manifold

| Specifications | frequency band | requirements | Comments |
| --- | --- | --- | --- |
| Average Insertion Loss | CF ±118MHz | ≤9.5dB |  |
| IL. Var. over temperature |  | ≤1.5dB |  |
| IL. Var. over life |  | ≤0.1dB | Non-measurable |
| IL. Var. | CF ±118MHz | ≤1.26dB |  |
| IL. Var | CF ±118MHz | ≤0.15dB/MHz |  |
| GD. Var. | CF ±110.8MHz | ≤7.5ns |  |
| GD. Var. | CF ±110.8MHz | ≤0.32ns/MHz |  |
| GD. Var. | CF ±118MHz | ≤9.9ns |  |
| GD. Var. | CF ±118MHz | ≤0.54ns/MHz |  |
| Av. Rejection | Adjacent Channel | ≥ 23dB |  |
| CPRL. | CF ± 118MHz | ≥16dB |  |
| output RL. | CF ± 118sMHz | ≥5.5dB |  |
| temp range | -20 to +60 |  |  |

Note: The definition of the 'Average rejection ':

- Calculate the average insertion loss of the passband:

(1)

where Nf is the number of points of the measured results in the predefined pass-band, represent the measured insertion loss at the frequency index i.

- Similarly, the average adjacent rejection is defined as:

(2)

where, Nf1is the number of points of the measured results in the predefined adjacent lower band, and Nf2 is that of the upper band. and are the average rejection level of the adjacent channels as equation 2 shown. Subsequently, the average rejection can be obtained by the average isolation subtracting the average insertion loss of the pass-bandcalculated by equation1. As is shown below:

(3)

测试结果

Figure2 to Figure4 illustrated the measured results of the WR34 manifold attached with new channel filters.

Figure2 WR34 Manifold Transfer and CPRL.

Figure3 WR34 channel filter Input Return Loss

Figure4 WR34 channel filter Group Delay Variation

Table3 summarized the compliance of the WR34 testing results:

Table1 WR34 Testing Results

| PN:H016B29A0002 | Specification |  | Measured |  |  | Compliance | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | -20℃ | 25℃ | 60 |  |  |
| Channel 1 | IL. | ≤9.5dB | -6.70 | -6.72 | -7.15 | pass |  |
|  | ILvar. | ≤0.5dB | 0.45 |  |  | pass |  |
|  | ILrip. | ≤1.5dB | 1.13 | 0.78 | 0.77 | pass |  |
|  | ILripR. | ≤0.15dB/MHz | 0.14 | 0.11 | 0.09 | pass |  |
|  | GDrip1. | ≤7.5ns | 5.62 | 4.94 | 4.98 | pass |  |
|  | GDripR1. | ≤0.32ns/MHz | 0.31 | 0.34 | 0.60 | pass |  |
|  | GDrip2. | ≤9.9ns | 8.92 | 7.43 | 7.24 | pass |  |
|  | GDripR2. | ≤0.54ns/MHz | 0.22 | 0.24 | 0.26 | pass |  |
|  | RJ. | ≥ 23dB | 23.53 | 25.52 | 26.45 | pass |  |
|  | RL. | ≥5.5dB | 5.98 | 6.04 | 5.85 | fail |  |
|  | CPRL. | ≥16dB | 19.44 | 17.95 | 17.21 | pass |  |
| Channel 2 | IL. | ≤9.5dB | 6.72 | 6.71 | 6.71 | pass |  |
|  | ILvar. | ≤0.5dB | 0.01 |  |  | pass |  |
|  | ILrip. | ≤1.5dB | 1.30 | 1.02 | 0.91 | pass |  |
|  | ILripR. | ≤0.15dB/MHz | 0.09 | 0.08 | 0.08 | pass |  |
|  | GDrip1. | ≤7.5ns | 8.20 | 4.65 | 4.90 | pass |  |
|  | GDripR1. | ≤0.32ns/MHz | 0.19 | 0.40 | 0.37 | pass |  |
|  | GDrip2. | ≤9.9ns | 4.96 | 7.39 | 6.94 | pass |  |
|  | GDripR2. | ≤0.54ns/MHz | 0.35 | 0.22 | 0.27 | pass |  |
|  | RJ. | ≥ 23dB | 26.04 | 26.28 | 26.35 | pass |  |
|  | RL. | ≥5.5dB | 5.77 | 6.13 | 5.91 | fail |  |
|  | CPRL. | ≥16dB | 18.09 | 17.64 | 16.98 | pass |  |
| Channel 3 | IL. | ≤9.5dB | 6.45 | 6.61 | 6.78 | pass |  |
|  | ILvar. | ≤0.5dB | 0.33 | 0.00 | 0.00 | pass |  |
|  | ILrip. | ≤1.5dB | 1.27 | 1.08 | 1.00 | pass |  |
|  | ILripR. | ≤0.15dB/MHz | 0.09 | 0.07 | 0.07 | pass |  |
|  | GDrip1. | ≤7.5ns | 4.71 | 5.18 | 5.58 | pass |  |
|  | GDripR1. | ≤0.32ns/MHz | 0.38 | 0.27 | 0.30 | pass |  |
|  | GDrip2. | ≤9.9ns | 7.61 | 7.33 | 7.98 | pass |  |
|  | GDripR2. | ≤0.54ns/MHz | 0.24 | 0.27 | 0.30 | pass |  |
|  | RJ. | ≥ 23dB | 26.51 | 26.65 | 26.56 | pass |  |
|  | RL. | ≥5.5dB | 5.93 | 5.93 | 5.91 | fail |  |
|  | CPRL. | ≥16dB | 18.12 | 19.22 | 19.82 | pass |  |
| Channel 4 | IL. | ≤9.5dB | 6.79 | 6.79 | 6.74 | pass |  |
|  | ILvar. | ≤0.5dB | 0.06 | 0.00 | 0.00 | pass |  |
|  | ILrip. | ≤1.5dB | 1.26 | 1.04 | 0.92 | pass |  |
|  | ILripR. | ≤0.15dB/MHz | 0.07 | 0.07 | 0.06 | pass |  |
|  | GDrip1. | ≤7.5ns | 4.89 | 4.98 | 5.05 | pass |  |
|  | GDripR1. | ≤0.32ns/MHz | 0.32 | 0.40 | 0.45 | pass |  |
|  | GDrip2. | ≤9.9ns | 7.53 | 6.89 | 7.00 | pass |  |
|  | GDripR2. | ≤0.54ns/MHz | 0.23 | 0.22 | 0.29 | pass |  |
|  | RJ. | ≥ 23dB | 26.05 | 26.20 | 26.28 | pass |  |
|  | RL. | ≥5.5dB | 5.30 | 5.44 | 5.65 | fail |  |
|  | CPRL. | ≥16dB | 18.00 | 18.28 | 19.12 | pass |  |
| Channel 5 | IL. | ≤9.5dB | 6.22 | 6.40 | 6.58 | pass |  |
|  | ILvar. | ≤0.5dB | 0.35 | 0.00 | 0.00 | pass |  |
|  | ILrip. | ≤1.5dB | 1.36 | 1.01 | 0.86 | pass |  |
|  | ILripR. | ≤0.15dB/MHz | 0.09 | 0.08 | 0.08 | pass |  |
|  | GDrip1. | ≤7.5ns | 4.79 | 4.44 | 4.37 | pass |  |
|  | GDripR1. | ≤0.32ns/MHz | 0.19 | 0.22 | 0.21 | pass |  |
|  | GDrip2. | ≤9.9ns | 6.96 | 6.06 | 5.97 | pass |  |
|  | GDripR2. | ≤0.54ns/MHz | 0.13 | 0.15 | 0.13 | pass |  |
|  | RJ. | ≥ 23dB | 23.71 | 23.91 | 23.85 | pass |  |
|  | RL. | ≥5.5dB | 4.98 | 5.38 | 5.62 | fail |  |
|  | CPRL. | ≥16dB | 17.30 | 18.88 | 20.49 | pass |  |
| Channel 6 | IL. | ≤9.5dB | 6.68 | 6.53 | 6.52 | pass |  |
|  | ILvar. | ≤0.5dB | 0.17 | 0.00 | 0.00 | pass |  |
|  | ILrip. | ≤1.5dB | 1.15 | 0.85 | 0.82 | pass |  |
|  | ILripR. | ≤0.15dB/MHz | 0.13 | 0.09 | 0.07 | pass |  |
|  | GDrip1. | ≤7.5ns | 5.45 | 4.90 | 4.95 | Pass |  |
|  | GDripR1. | ≤0.32ns/MHz | 0.38 | 0.54 | 0.57 | Pass |  |
|  | GDrip2. | ≤9.9ns | 8.36 | 7.28 | 7.19 | pass |  |
|  | GDripR2. | ≤0.54ns/MHz | 0.19 | 0.23 | 0.24 | pass |  |
|  | RJ. | ≥ 23dB | 43.97 | 44.29 | 44.14 | pass |  |
|  | RL. | ≥5.5dB | 5.19 | 5.38 | 5.55 | fail |  |
|  | CPRL. | ≥16dB | 21.00 | 25.18 | 23.06 | pass |  |
| Channel 7 | IL. | ≤9.5dB | 6.32 | 6.34 | 6.56 | pass |  |
|  | ILvar. | ≤0.5dB | 0.24 | 0.00 | 0.00 | pass |  |
|  | ILrip. | ≤1.5dB | 0.95 | 1.29 | 0.85 | pass |  |
|  | ILripR. | ≤0.15dB/MHz | 0.15 | 0.12 | 0.08 | pass |  |
|  | GDrip1. | ≤7.5ns | 6.08 | 5.79 | 5.19 | Pass |  |
|  | GDripR1. | ≤0.32ns/MHz | 0.29 | 0.32 | 0.34 | Pass |  |
|  | GDrip2. | ≤9.9ns | 9.04 | 8.25 | 7.17 | pass |  |
|  | GDripR2. | ≤0.54ns/MHz | 0.20 | 0.20 | 0.23 | pass |  |
|  | RJ. | ≥ 23dB | 43.73 | 45.17 | 45.61 | pass |  |
|  | RL. | ≥5.5dB | 5.33 | 4.87 | 5.09 | fail |  |
|  | CPRL. | ≥16dB | 17.62 | 18.10 | 19.04 | Pass |  |
| Channel 8 | IL. | ≤9.5dB | 6.79 | 6.78 | 6.74 | Pass |  |
|  | ILvar. | ≤0.5dB | 0.05 | 0.00 | 0.00 | Pass |  |
|  | ILrip. | ≤1.5dB | 0.80 | 1.16 | 0.77 | Pass |  |
|  | ILripR. | ≤0.15dB/MHz | 0.14 | 0.08 | 0.05 | Pass |  |
|  | GDrip1. | ≤7.5ns | 5.59 | 5.14 | 4.62 | Pass |  |
|  | GDripR1. | ≤0.32ns/MHz | 0.24 | 0.24 | 0.43 | Pass |  |
|  | GDrip2. | ≤9.9ns | 8.80 | 7.75 | 6.69 | Pass |  |
|  | GDripR2. | ≤0.54ns/MHz | 0.17 | 0.16 | 0.22 | Pass |  |
|  | RJ. | ≥ 23dB | 25.58 | 24.89 | 24.66 | Pass |  |
|  | RL. | ≥5.5dB | 5.30 | 4.85 | 5.15 | fail |  |
|  | CPRL. | ≥16dB | 19.19 | 19.22 | 19.52 | Pass |  |

list of symbols:

IL.       = Average Insertion Loss

ILvar.     = Insertion Loss variation over tempreature

ILrip.     = Insertion Loss ripple in CF +/-118MHz

ILripR.    = Insertion Loss ripple rate in CF +/-118MHz

GDrip1.   = Group delay variation in CF +/-110.8MHz

GDripR1  =Group delay variation rate in CF +/-110.8MHz

GDrip2.   = Group delay variation in CF +/-118MHz

GDripR2  =Group delay variation rate in CF +/-118MHz

RJ.      =Average Adjacent Channel Rejection across the passband of the adjacent channel

CPRL.    =Common Port RL CF +/- 118MHz

RL.      =output Port Return Loss CF +/- 118MHz

软件需求：

- 根据输入的8个s2p文件输出曲线

- 根据输入的8个s2p文件填写上述表格，给出pass/fail

- 根据实时测试结果在调试过程中实时判定是否满足指标

参考：

Matlab代码